Adding the MCS7703 drivers to the Platform

Introduction:

This document provides the information about how to add/include MCS7703 device drivers to a windows CE .NET 4.2 platform.

The following steps explains how to add/include the MCS7703 device drivers to Windows CE .NET 4.2 platform.

- Create and Build a CEPC platform

- After building the platform add the following entries in the platform.reg file

[HKEY_LOCAL_MACHINE\Drivers\USB\LoadClients\38672_30467\255_0_255\Default\

Mos03UsbSerialClass]

"Dll"="USer03Ce.DLL"

[HKEY_LOCAL_MACHINE\Drivers\USB\ClientDrivers\Mos03UsbSerialClass]

"Dll"="USer03Ce.DLL"

- Add the following entries in the platform.bib file

USer03Ce.dll      $(_FLATRELEASEDIR)\User03Ce.dll             NK  SH

- Copy the USer03Ce.dll in to the following folder.

Drive:>\WINCE420\PUBLIC\CEPlatform\RelDir\CEPC_X86Release

- Then build the platform to effect the above changes.

This platform image contains the MCS7703 device driver and you can connect MCS7703 to the target machine and configure after downloading the platform OS image.