# MosUsbCe

# USB to Serial Device Driver

# For Windows CE .NET

MosChip Semiconductor Technology Ltd

Version 0.1, October 26, 2004

- Contents

1. Introduction	1

2. Resource Requirements	1

3. Install the Driver	1

4. Test the device with the direct cable connection	2

## 1. Introduction

- This document provides the information about how to install the driver for MOSCHIP MCS 7703 USB2Serial device on windows CE .NET 4.2 environments using the CAB file.

## 2. Resource Requirements

- For installation of the driver using .cab file, the following resources are needed:

- CEPC/Windows CE Embedded Device with Windows CE 4.2 OS

- PC having Win 2k with Microsoft ActiveSync.

- NULL Modem Cable (internally connected strictly according to RS232 standard)

- MCS 7703 USB2Serial device.

## 3. Install the Driver

The Installation of the MCS7703 Driver by a .cab file can be done through the following steps:

- In the PC, Microsoft ActiveSync should be installed.

- Connect the WinCE Device (CEPC) and the PC through a Null modem cable.

- Run ActiveSync in the PC.

- Open the Command Prompt in WinCE Device and type the command ‘repllog’ and press Enter.

- A New Partnership window appears in ActiveSync. Select No and then click on Next.

- Then it will establish a connection between PC and WinCe Device and it will show the connection status as Connected on ActiveSync.

- Then click on the Explore button on the ActiveSync window, and copy the USer03Ce WinZip (it is actually a CAB file) file to the Mobile Device folder on your PC.

- Then open My Computer in the WinCe Device (CEPC) and the CAB file (User03Ce) will be there.

- To install the driver, just double click on the CAB file (User03Ce).

- An installation window will appear. Click OK to install. The driver is now installed.

- Now, insert the MCS 7703 USB2Serial device in to USB port on the WinCE Device. The driver will be loaded and the device is ready for connection to other machines.

## 4. Test the device with the direct cable connection

- After plugging the device on the USB port, click on the Start button.

- Go to Settings and click on the Network and Dial-up Connections

- Double click on the Make New Connection.

- Select Direct Connection and Press Next.

- Select the Device Usb To Serial COM

- Press TCP/IP Settings.

- Uncheck the Use server-assigned IP address, Use Software Compression and Use IP header compression. And give the IP address (e.g. 192.168.1.1) and click on OK.

- Connect the MCS7703 serial port to standard serial port of the other machine using the Null Modem cable. Configure the other machine serial port for accepting the incoming connection.

- Double click the My Connection to establish the direct cable connection.

- Dialog box appears which tells the connection status/progress.

- Message box displays “Connected” when the communication is established.

- After Run the Ping command to test the hardware.