Adding the MosUsbCe drivers to the Platform

Introduction:

This document provides the information about how to add/include MosUsbCe drivers to a windows CE .NET 4.2 platform.

The following steps explains how to add/include the MosUsbCe drivers to Windows CE .NET 4.2 platform.

- Build your platform

- After building the platform add the following entries in the platform.reg file

[HKEY_LOCAL_MACHINE\Drivers\USB\LoadClients\38672_30768\255_0_255\Default\mosusbce]

"Dll"="mosusbce.DLL"

[HKEY_LOCAL_MACHINE\Drivers\USB\ClientDrivers\Default]

"Dll"="mosusbce.DLL"

- Add the following entries in the platform.bib file

mosusbce.dll      $(_FLATRELEASEDIR)\mosusbce.dll             NK  SH

- Copy the MosUsbCe.dll in to the following folder.

Drive :> \WINCE420\PUBLIC\CEPlatform\RelDir\CEPC_X86Release

- Then build the platform to effect the above changes.

This platform image contains the MosUsbCe driver and you can connect MCS7730 to the target machine and configure after downloading the platform OS image.