a) Procedure for Win 2000 / XP

Sequence of Operation to be done to override the MAC address

- Connect the MCS7830 board which pre-programmed with Permanent MAC address into EEPROM. Test & ensure Ping.

- Go to the PropertySheet of Driver ( Right click on MCS7830 entry in Device Manager to go to property sheet ) & Click on Present radio button and enter

a  valid MAC address into “Network Address” field.

- Use 'ipconfig /all' and check for the New MAC Address. Do the Ping Test.

- Go to the PropertySheet again and in the 'NetworkAddress' Field click on 'Not Present' Option., to revert back to Permanent MAC address.

- Use 'ipconfig/all' and check for the Permanent MAC Address. Do the Ping Test.

Limitation

Let me explain the Limitation of this approach with an Example.

Assume we have two MCS7830 boards.

MCS7830_1 : Permanent MAC Address 00-50-C2-1B-70-70

MCS7830_2 : Permanent MAC Address 00-50-C2-1B-70-71

The User Installs MCS7830_1. Permanent MAC Address is written into Registry. He overrides with some other Local MAC Address. Without UnInstalling the Driver, if the User puts mCS7830_2, the Driver will think that the Permanent Address which is already written for MCS7830_1 is same for MCS7830_2 also. And the MAC address MCS7830_1 would be written into MCS7830_2 EEPROM.

Hence MCS7830_2 will loose the  Permanent MAC Address of 00-50-C2-1B-70-71 from its EEprom.

To overcome above limitation, one needs to Un-install & run the setup to re-Install the Driver.

Note : Procedure as stated above is same Win 98SE / ME also except for the step 4. ( I.e reverting to Permanent MAC address )

For reverting to Permanent MAC address in Win 98SE / ME, goto windows registry and use the Find option and search for Local address key under :

HKEY_LOCAL_MACHINE / currentcontrolset / class / Net

(Eg. 0050C21B7080, let us assume this was set as Local address) & delete the specific key. The unplug / re-plug the device to revert to Permanent MAC address.